Micromouse Summary Report

Dalton Hanna, Jorge Sosa

4/30/2026

Overview

The goal of this project was to design and build an autonomous micromouse robot capable of navigating and solving a maze. The robot was designed with a focus on low cost, compact size, and reliable navigation using sensors and control logic. The system integrates mechanical design, electronics, and programming to achieve autonomous movement and decision-making.

Chassis Design

The chassis was inspired by some professional micromouse chassis, but it was ultimately custom-made. The design was achieved by using AutoCAD, which was then 3d printed to meet the size constraints of the competition. The design was optimized to fit the maze pathways while maintaining stability and proper weight distribution

Key design considerations:

- Mounting points for the motors and caster wheel

- Space for batteries and wiring

- Wheel clearance and alignment

- Low center of gravity for stability

The final chassis design and motor mounts were printed using PLA, which allowed for easy assembly and modification

Sensors Used

The robot uses three infrared reflective sensors (TCRT5000) to detect walls and navigate the maze. These sensors output analog values that are read by the Arduino and converted into distance for wall detection thresholds.

Sensor placement:

- Left sensor – to detect walls on the left (perpendicular to the walls)

- Front sensor – to detect obstacles ahead

- Right sensor – to detect walls on the right (perpendicular to the walls)

Drive Motors

The micro mouse is powered by DC gear motors with encoders (N20-type motors)

Key features:

- Compact size for a small chassis

- Gear reduction for high torque

- Encoders for speed and position feedback

All motors are connected to press-fit wheels and controlled through a motor driver to allow forward, reverse, and turning motion.

Controller Device

The main controller we went with is an Arduino Nano

Key reasons

- Small form factor (fits on a half-size breadboard with enough room for the motor driver and wires)

- Enough I/O pins for sensors and motors

- Easy programming and debugging

The Arduino reads sensor inputs, processes navigation logic, and outputs control signals to motors

Maze-solving strategy

The robot uses a simple but effective wall-forming algorithm (right-hand rule)

- If there is no wall on the right, turn right

- If the front is clear, move forward

- If blocked, turn left or reverse

This ensures the robot can explore and eventually reach the end of the maze, even without mapping

Decision Making

The robot determines which way to turn based on real-time sensor readings

- IR sensor values are compared to thresholds

- Based on detecting walls, the program selects movement

  - Turn right

  - Move forward

  - Turn left

The logic runs continuously in a loop, allowing the robot to adapt dynamically.

Stopping behavior

Yes, the robot stops when required. It stops when:

- A wall is detected directly in front, and no immediate path is available

- It reaches the goal condition (end of the maze)

Stopping is implemented by simply setting the motor speeds to zero

Handling Dead Ends

The robot can detect and escape dead ends:

- If front, left, and right are all blocked, that means it’s a dead end

- The robot performs a 180° turn

- It then moves back and continues navigation

Turning without getting stuck

The robot uses controlled motor timing and differential drives to turn smoothly:

- One motor forward, one backward gives us a pivot turn

- Timed turns ensure consistent angles (90° or 180°)

- Proper wheel alignment and chassis design reduce friction issues

Detecting the end of the maze

The robot determines the end of the maze based on:

- Lack of surrounding walls (open space condition), or

- Reaching a predefined goal location in the logic

In this implementation, the robot stops when it reaches an area that satisfies the condition defined in the code

Program Overview

The micromouse was programmed using a right-hand-following algorithm to navigate the maze. This method prioritizes keeping the robot's right side in contact with a wall, allowing it to systematically explore the maze without requiring prior knowledge of the layout

Decision points

- If the right path is clear, turn right and move forward

- If the right path is blocked but the front is clear, continue forward

- If both the right and front are blocked, turn left

- If all directions are blocked, turn around

This approach ensures continuous movement while maintaining consistent reference, helping the robot avoid getting lost. Although simple, the algorithm is effective for solving mazes without isolated loops and provides a reliable baseline for autonomous navigation

Conclusion

This micromouse project demonstrates the integration of mechanical design, electronics, and embedded programming to create an autonomous system. While the current implementation uses a basic wall-following algorithm, it successfully navigates the maze and handles obstacles such as dead ends