Micromouse Full Code

// ============================================================

//  Maze Robot — Right Hand Rule

//  Priority: RIGHT → FORWARD → LEFT → 180

//  Sensors: LOW reading = wall, HIGH reading = open

// ============================================================

// ────────────────────────────────────────────────────────────

//  PINS

// ────────────────────────────────────────────────────────────

const int startButton = 12;

const int PWMA = 6;  const int AIN1 = 7;  const int AIN2 = 8;

const int PWMB = 9;  const int BIN1 = 10; const int BIN2 = 11;

const int leftSensor  = A0;

const int frontSensor = A1;

const int rightSensor = A2;

// ────────────────────────────────────────────────────────────

//  SPEEDS

// ────────────────────────────────────────────────────────────

const int driveL  = 150;

const int driveR  = 57;

const int turnL   = 150;

const int turnR   = 80;

// ────────────────────────────────────────────────────────────

//  THRESHOLDS

//  Your sensors: wall = 200-400 (LOW), open = 600+ (HIGH)

//  WALL_THRESHOLD: below this = wall, above this = open

//  STOP_THRESHOLD: front sensor stops the robot when below this

// ────────────────────────────────────────────────────────────

const int WALL_THRESHOLD = 150;

const int STOP_THRESHOLD = 200;

// ────────────────────────────────────────────────────────────

//  TIMING  (all in milliseconds)

// ────────────────────────────────────────────────────────────

const int TURN_RIGHT_MS = 450;   // time for 90 degree right turn

const int TURN_LEFT_MS  = 900;   // time for 90 degree left turn  <- tune separately if needed

const int TURN_180_MS   = 900;  // time for 180 degree turn

const int BACKUP_MS     = 200;   // back up after hitting front wall

const int CORRECTION    = 12;    // steering correction strength

// ────────────────────────────────────────────────────────────

//  CENTER DETECTION & STALL

// ────────────────────────────────────────────────────────────

const int MOVES_TO_CENTER       = 20;

const unsigned long STALL_MS    = 10000UL;

// ────────────────────────────────────────────────────────────

//  PATH LOG

// ────────────────────────────────────────────────────────────

const int MAX_PATH = 400;

int  turnLog[MAX_PATH];

int  pathLen = 0;

// ────────────────────────────────────────────────────────────

//  STATE

// ────────────────────────────────────────────────────────────

bool          started      = false;

bool          returning    = false;

int           moveCount    = 0;

int           heading      = 0;

unsigned long lastMoveTime = 0;

// ============================================================

//  MOTORS

// ============================================================

void leftFwd(int s)  { digitalWrite(BIN1, LOW);  digitalWrite(BIN2, HIGH); analogWrite(PWMB, s); }

void leftRev(int s)  { digitalWrite(BIN1, HIGH); digitalWrite(BIN2, LOW);  analogWrite(PWMB, s); }

void rightFwd(int s) { digitalWrite(AIN1, HIGH); digitalWrite(AIN2, LOW);  analogWrite(PWMA, s); }

void rightRev(int s) { digitalWrite(AIN1, LOW);  digitalWrite(AIN2, HIGH); analogWrite(PWMA, s); }

void stopMotors() {

digitalWrite(BIN1, HIGH); digitalWrite(BIN2, HIGH); analogWrite(PWMB, 255);

digitalWrite(AIN1, HIGH); digitalWrite(AIN2, HIGH); analogWrite(PWMA, 255);

delay(40);

analogWrite(PWMB, 0); analogWrite(PWMA, 0);

digitalWrite(BIN1, LOW); digitalWrite(BIN2, LOW);

digitalWrite(AIN1, LOW); digitalWrite(AIN2, LOW);

}

// ============================================================

//  SENSORS

//  wall  = reading BELOW WALL_THRESHOLD

//  open  = reading ABOVE WALL_THRESHOLD

// ============================================================

bool hasWall(int pin) {

long total = 0;

for (int i = 0; i < 5; i++) { total += analogRead(pin); delay(2); }

return (total / 5) < WALL_THRESHOLD;

}

// ============================================================

//  HEADING

// ============================================================

void headRight()  { heading = (heading + 1) % 4; }

void headLeft()   { heading = (heading + 3) % 4; }

void headAround() { heading = (heading + 2) % 4; }

// ============================================================

//  TURNS — separate timing for left and right

// ============================================================

void doRight() {

Serial.println("TURN RIGHT");

leftFwd(turnL); rightRev(turnR);

delay(TURN_RIGHT_MS);

stopMotors(); delay(150);

headRight();

}

void doLeft() {

Serial.println("TURN LEFT");

leftRev(turnL); rightFwd(turnR);

delay(TURN_LEFT_MS);

stopMotors(); delay(150);

headLeft();

}

void do180() {

Serial.println("180");

leftFwd(turnL); rightRev(turnR);

delay(TURN_180_MS);

stopMotors(); delay(150);

headAround();

}

// ============================================================

//  DRIVE FORWARD

//  Drives until front sensor reads below STOP_THRESHOLD.

//  Wall correction keeps robot straight.

//  Backs up BACKUP_MS after stopping.

// ============================================================

void driveToWall() {

leftFwd(driveL);

rightFwd(driveR);

while (analogRead(frontSensor) > STOP_THRESHOLD) {

int L = analogRead(leftSensor);

int R = analogRead(rightSensor);

int lPWM = driveL;

int rPWM = driveR;

if (L < WALL_THRESHOLD && R < WALL_THRESHOLD) {

int err = L - R;

lPWM += err / 6;

rPWM -= err / 6;

} else if (R < WALL_THRESHOLD) {

lPWM -= CORRECTION; rPWM += CORRECTION;

} else if (L < WALL_THRESHOLD) {

lPWM += CORRECTION; rPWM -= CORRECTION;

}

leftFwd(constrain(lPWM, 80, 200));

rightFwd(constrain(rPWM, 20, 100));

delay(10);

}

stopMotors(); delay(100);

leftRev(driveL); rightRev(driveR);

delay(BACKUP_MS);

stopMotors(); delay(100);

}

// ============================================================

//  SETUP

// ============================================================

void setup() {

pinMode(startButton, INPUT_PULLUP);

pinMode(PWMA, OUTPUT); pinMode(AIN1, OUTPUT); pinMode(AIN2, OUTPUT);

pinMode(PWMB, OUTPUT); pinMode(BIN1, OUTPUT); pinMode(BIN2, OUTPUT);

stopMotors();

Serial.begin(9600);

Serial.println("=== CALIBRATION (3s) ===");

unsigned long t = millis() + 3000;

while (millis() < t) {

int L = analogRead(leftSensor);

int F = analogRead(frontSensor);

int R = analogRead(rightSensor);

Serial.print("L:"); Serial.print(L);

Serial.print(" F:"); Serial.print(F);

Serial.print(" R:"); Serial.print(R);

Serial.print(" | L="); Serial.print(L < WALL_THRESHOLD ? "WALL" : "open");

Serial.print(" F="); Serial.print(F < WALL_THRESHOLD ? "WALL" : "open");

Serial.print(" R="); Serial.println(R < WALL_THRESHOLD ? "WALL" : "open");

delay(300);

}

Serial.println("=== READY ===");

}

// ============================================================

//  LOOP

// ============================================================

void loop() {

if (!started) {

if (digitalRead(startButton) == LOW) {

delay(50);

if (digitalRead(startButton) == LOW) {

started = true; returning = false;

moveCount = 0; pathLen = 0;

heading = 0; lastMoveTime = millis();

delay(500);

}

}

return;

}

// Stall escape

if (!returning && millis() - lastMoveTime > STALL_MS) {

Serial.println("STALL");

leftRev(driveL); rightRev(driveR); delay(400); stopMotors();

do180();

lastMoveTime = millis();

if (pathLen < MAX_PATH) turnLog[pathLen++] = 2;

return;

}

if (returning) returnStep();

else           solveStep();

}

// ============================================================

//  SOLVE STEP

// ============================================================

void solveStep() {

if (moveCount >= MOVES_TO_CENTER) {

stopMotors();

Serial.println("CENTER");

delay(1000);

returning = true;

lastMoveTime = millis();

return;

}

// Drive to front wall and back up

// Drive to wall and stop — read sensors BEFORE backing up

leftFwd(driveL); rightFwd(driveR);

while (analogRead(frontSensor) > STOP_THRESHOLD) {

int Lv = analogRead(leftSensor);

int Rv = analogRead(rightSensor);

int lPWM = driveL, rPWM = driveR;

if (Lv < WALL_THRESHOLD && Rv < WALL_THRESHOLD) { int err = Lv - Rv; lPWM += err/6; rPWM -= err/6; }

else if (Rv < WALL_THRESHOLD) { lPWM -= CORRECTION; rPWM += CORRECTION; }

else if (Lv < WALL_THRESHOLD) { lPWM += CORRECTION; rPWM -= CORRECTION; }

leftFwd(constrain(lPWM,80,200)); rightFwd(constrain(rPWM,20,100));

delay(10);

}

stopMotors(); delay(100);

// Read sensors NOW while still close to front wall — most accurate

bool R = hasWall(rightSensor);

bool F = hasWall(frontSensor);

bool L = hasWall(leftSensor);

// Back up to give turn clearance

leftRev(driveL); rightRev(driveR);

delay(BACKUP_MS);

stopMotors(); delay(100);

moveCount++;

lastMoveTime = millis();

Serial.print("R="); Serial.print(R);

Serial.print(" F="); Serial.print(F);

Serial.print(" L="); Serial.println(L);

Serial.print("DECIDING: R="); Serial.print(R);

Serial.print(" F="); Serial.print(F);

Serial.print(" L="); Serial.print(L);

Serial.print(" !R="); Serial.print(!R);

Serial.print(" !F="); Serial.print(!F);

Serial.print(" !L="); Serial.println(!L);

int t = 0;

if (!R) {

Serial.println("BRANCH: RIGHT");

doRight(); t = 1;

} else if (!F) {

Serial.println("BRANCH: STRAIGHT");

t = 0;

} else if (!L) {

Serial.println("BRANCH: LEFT - EXECUTING NOW");

// Left turn: left motor reverse, right motor forward

digitalWrite(BIN1, HIGH); digitalWrite(BIN2, LOW);  analogWrite(PWMB, turnL);

digitalWrite(AIN1, HIGH); digitalWrite(AIN2, LOW);  analogWrite(PWMA, turnR);

delay(TURN_LEFT_MS);

stopMotors(); delay(150);

headLeft();

t = 3;

} else {

Serial.println("BRANCH: 180");

// 180: same as right turn but longer

digitalWrite(BIN1, LOW);  digitalWrite(BIN2, HIGH); analogWrite(PWMB, turnL);

digitalWrite(AIN1, LOW);  digitalWrite(AIN2, HIGH); analogWrite(PWMA, turnR);

delay(TURN_180_MS);

stopMotors(); delay(150);

headAround();

t = 2;

}

if (pathLen < MAX_PATH) turnLog[pathLen++] = t;

delay(100);

}

// ============================================================

//  RETURN STEP

// ============================================================

void returnStep() {

if (pathLen == 0) {

stopMotors();

started = false; returning = false;

Serial.println("HOME");

return;

}

leftFwd(driveL); rightFwd(driveR);

delay(800);

stopMotors(); delay(100);

int t = turnLog[--pathLen];

if      (t == 1) { doLeft();  }

else if (t == 3) { doRight(); }

else if (t == 2) { do180();   }

lastMoveTime = millis();

Serial.print("steps left:"); Serial.println(pathLen);

delay(100);

}